Here is the comprehensive report covering the work completed over the last 10 days, including the system integration, AI implementation, and the critical debugging of the hand interaction system.

# Sprint Report: AR Control System & AI Integration (MVP)

Project: XR Line Assistant (HoloLens 2)

Duration: Last 10 Days

Status: Deployed & Functional

### 1. Core System Integration (Node-RED + Unity + PLC)

We successfully established a real-time data link between the physical machine layer and the AR headset.

- Data Bridge: Implemented ApiDataManager.cs to fetch JSON data from the Node-RED middleware (connected to the PLC).

- Station Visualization: Created StationInfoDisplay.cs to visualize live machine states (Idle/Running), recipe codes, and Hopper levels (Low/Full) directly on UI panels.

- Feeder Visualization: Developed FeederDisplay.cs to map integer values (cap counts) to dynamic UI fill bars and floating 3D text labels above the physical feeder stations.

### 2. AI Computer Vision Implementation

We integrated a computer vision loop to allow the HoloLens to "see" and classify objects.

- Image Capture: Built HoloLensCameraCapture.cs to access the HoloLens PV (Photo/Video) camera.

- Server Communication: Configured the app to encode images to JPG and transmit them wirelessly to a local Python/FastAPI server.

- Result Parsing: The system now parses the JSON response to detect "Round" vs. "Square" caps and displays the classification result in the user's view in real-time.

### 3. Immersive Deployment Configuration

We transitioned the project from a "Holographic Remoting" testing state to a standalone device build.

- Windowed Mode Fix: Resolved the issue where the app launched as a 2D "Slate" (window) by enabling "Initialize XR on Startup" in Project Settings.

- Feature Conflicts: Resolved build errors by disabling the "Holographic Remoting" feature group when building for the device, ensuring the app runs natively on the HoloLens hardware.

### 4. Critical Bug Fix: Hand Interaction & Input

Issue:

Users could "Air Tap" and click buttons inside the Unity Editor (Holographic Remoting), but hand inputs failed completely (no click response) after deploying to the physical HoloLens device.

Root Cause:

The OpenXR Interaction Profiles were missing from the Build Settings. The Unity Editor uses a simulation layer that bypasses these profiles, but the physical hardware requires explicit definitions to map "Pinch" gestures to "Select" actions.

Resolution:

- Navigated to Project Settings > XR Plug-in Management > OpenXR (Windows Tab).

- Added the Microsoft Hand Interaction Profile.

- (Optimization) Removed the generic "Hand Interaction Profile" to ensure native HoloLens gesture support.

Outcome:

Air-tapping, pinching, and system gestures now function correctly on the standalone build.

### 5. UX Enhancement: Movable Workspace

Requirement:

The user required the ability to reorganize floating information panels (Visualizers) to prevent view blocking, while ensuring the heavy machinery (Digital Twin) and Main Menu remained fixed in place.

Implementation:

We applied a component-based architecture to specific GameObjects:

- Target Objects: Visualizer_Round and Visualizer_Square.

- Components Added:

  - Box Collider (To define the grabbable area).

  - ObjectManipulator (To handle the physics of moving/rotating).

  - NearInteractionGrabbable (To allow direct hand manipulation).

- Static Objects: Explicitly excluded SIF402 (Machine) and StartupMenu from these scripts to keep them anchored.

### System Status

The application is now a fully integrated AR Control Interface. It visualizes live data from the factory floor, uses AI to verify inventory, and offers a flexible, immersive user interface that fully utilizes HoloLens 2 hand tracking capabilities.